- Carla学习记录

- 基础知识

- Client

Carla是一个bs架构，服务端负责渲染，客户端负责获取服务端的渲染结果；因此显而易见，各类API的调用都会出现同步/异步问题；

host = '10.10.32.21'
port=2000
client = carla.Client(host,port)
client.set_timeout(2.0)

- World

可以理解为所有渲染场景的容器，所有的actor的添加，销毁，获取入口

world = client.get_world()

- Settings

settings = world.get_settings()

获取整个世界的设置；

settings.synchronous_mode = True
settings.fixed_delta_seconds = 0.05

world.apply_settings(settings)

- traffic_manager（可以声明多个）

https://blog.csdn.net/qq_39537898/article/details/117562578

所有交通对象的管理类，车辆，行人都由此获取

traffic_manager = client.get_trafficmanager(8000)

traffic_manager.set_synchronous_mode(True) 设置为同步模式，并且world的设置也要设置为同步，否则碰撞计算就会全部错误；

traffic_manager.set_global_distance_to_leading_vehicle(2.5) 设置与前车的距离

注册：Vehicle Registry

计算：Localization Stage， Collision Stage.Traffic Light Stage.Motion Planner.

- 获取相应的蓝图类

bps = world.get_blueprint_library().filter(filter)

获取类之后，即可用已存在的类进行实例的创建

已知的filter：

VEHICLE='vehicle.*'
WALKER="walker.pedestrian.*"

- 获取可放置车辆的点位（带transform）

spawn_points = world.get_map().get_spawn_points()

- SpawnActor

第一个参数为对象类型，第二个参数为放置的位置

初始化一个蓝图类SpawnActor(blueprint, transform)

- 初始化设置车辆

设置车辆的颜色，id，是否为hero，添加自动模式

blueprint = random.choice(blueprints)
if blueprint.has_attribute('color'):
    color = random.choice(blueprint.get_attribute('color').recommended_values)
    blueprint.set_attribute('color', color)
if blueprint.has_attribute('driver_id'):
    driver_id = random.choice(blueprint.get_attribute('driver_id').recommended_values)
    blueprint.set_attribute('driver_id', driver_id)
if hero:
    #如果是hero代表为主控，否则为自动驾驶的车辆
    blueprint.set_attribute('role_name', 'hero')
    hero = False
else:
    blueprint.set_attribute('role_name', 'autopilot')
    # spawn the cars and set their autopilot and light state all together
item = SpawnActor(blueprint, transform)
             .then(SetAutopilot(FutureActor, True, traffic_manager.get_port()))
batch.append(item )

SetAutopilot

设置自动驾驶模式；以spawnActor的模式实例化，其中第一个参数为蓝图类，第二个参数为transform；

- 开启/关闭车辆的自动驾驶

SetAutopilot(FutureActor, True, traffic_manager.get_port())

- 批量添加车辆至场景中（同步）

for response in client.apply_batch_sync(batch, synchronous_master):
    if response.error:
        logging.error(response.error)
    else:
        vehicles_list.append(response.actor_id)

- 添加自行走行人（必须和aiworker关联使用）

walker绑定了一个父级；

SpawnActor(walker_controller_bp, carla.Transform(), walkers_list[i]["id"]

因此实现的逻辑为：

首先获取出walker中的行人类：

然后循环实例化，使用声明式transform设置其几何变换，地图随机选取点；

批量添加；

然后创建同样数量的aiwalker类，父级设定为人

再批量添加；

最后启动ai walker的行走即可；

- Weather

- Camera

获取相机蓝图类；

设置相机成像的宽高，设置fov，设置transform

camera_bp = world.get_blueprint_library().find('sensor.camera.rgb')
camera_bp.set_attribute("image_size_x", str(args.width))
camera_bp.set_attribute("image_size_y", str(args.height))
camera_bp.set_attribute("fov", str(args.fov))
camera = world.spawn_actor(camera_bp, carla.Transform())

其后，如添加自走行人的逻辑一直；添加进入场景

world.set_pedestrians_seed(1235)
ped_bp = random.choice(world.get_blueprint_library().filter("walker.pedestrian.*"))
trans = carla.Transform()
trans.location = world.get_random_location_from_navigation()
ped = world.spawn_actor(ped_bp, trans)
walker_controller_bp = world.get_blueprint_library().find('controller.ai.walker')
controller = world.spawn_actor(walker_controller_bp, carla.Transform(), ped)
controller.start()
controller.go_to_location(world.get_random_location_from_navigation())
controller.set_max_speed(1.7)

# keep tracking of actors to remove
actor_list.append(camera)
actor_list.append(ped)
actor_list.append(controller)

- 传感器观察类（用于场景返回数据）

获取传感器数据，实际上就是sensor_queue.get；

def _retrieve_data(self, sensor_queue, timeout):
    while True:
        data = sensor_queue.get(timeout=timeout)
        if data.frame == self.frame:
            return data
# ---------------

Sensor_queue是一个每帧渲染返回的数据

def make_queue(register_event):
    q = queue.Queue()
    register_event(q.put)
    self._queues.append(q)

make_queue(self.world.on_tick)

- 相机跟随actor

# move camera around
trans = ped.get_transform()
x = math.cos(turning) * -3
y = math.sin(turning) * 3
trans.location.x += x
trans.location.y += y
trans.location.z = 2
trans.rotation.pitch = -16
trans.rotation.roll = 0
trans.rotation.yaw = -360 * (turning/(math.pi*2))
camera.set_transform(trans)

图像数据转array

def get_image_as_array(image):
    array = np.frombuffer(image.raw_data, dtype=np.dtype("uint8"))
    array = np.reshape(array, (image.height, image.width, 4))
    array = array[:, :, :3]
    array = array[:, :, ::-1]
    # make the array writeable doing a deep copy
    array2 = copy.deepcopy(array)
    return array2

- 世界坐标转屏幕坐标

投影矩阵

def build_projection_matrix(w, h, fov):
    focal = w / (2.0 * np.tan(fov * np.pi / 360.0))
    K = np.identity(3)
    K[0, 0] = K[1, 1] = focal
    K[0, 2] = w / 2.0
    K[1, 2] = h / 2.0
    return K

get_screen_points(camera, K, image_w, image_h, points)

def get_screen_points(camera, K, image_w, image_h, points3d):
    
    # get 4x4 matrix to transform points from world to camera coordinates
    world_2_camera = np.array(camera.get_transform().get_inverse_matrix())

    # build the points array in numpy format as (x, y, z, 1) to be operable with a 4x4 matrix
    points_temp = []
    for p in points3d:
        points_temp += [p.x, p.y, p.z, 1]
    points = np.array(points_temp).reshape(-1, 4).T
    
    # convert world points to camera space
    points_camera = np.dot(world_2_camera, points)
    
    # New we must change from UE4's coordinate system to an "standard"
    # (x, y ,z) -> (y, -z, x)
    # and we remove the fourth component also
    points = np.array([
        points_camera[1],
        points_camera[2] * -1,
        points_camera[0]])
    
    # Finally we can use our K matrix to do the actual 3D -> 2D.
    points_2d = np.dot(K, points)

    # normalize the values and transpose
    points_2d = np.array([
        points_2d[0, :] / points_2d[2, :],
        points_2d[1, :] / points_2d[2, :],
        points_2d[2, :]]).T

    return points_2d

- 初始化Transform方式

camera_transform = carla.Transform(carla.Location(x=-5.5, z=2.8), carla.Rotation(pitch=-15))

- 添加lidar传感器

lidar_bp = bp_lib.filter("sensor.lidar.ray_cast")[0]

if args.no_noise:
    lidar_bp.set_attribute('dropoff_general_rate', '0.0')
    lidar_bp.set_attribute('dropoff_intensity_limit', '1.0')
    lidar_bp.set_attribute('dropoff_zero_intensity', '0.0')

#lidar的fov上顶30
lidar_bp.set_attribute('upper_fov', str(args.upper_fov))

#lidar的fov下限-25
lidar_bp.set_attribute('lower_fov', str(args.lower_fov))

#lidar线束
lidar_bp.set_attribute('channels', str(args.channels))
lidar_bp.set_attribute('range', str(args.range))

#每秒发射的点云数量，10万个
lidar_bp.set_attribute('points_per_second', str(args.points_per_second))

- 进阶

- Lidar_to_camera

初始化camera，vehicle，lidar

获取图像的输出

im_array = np.copy(np.frombuffer(image_data.raw_data, dtype=np.dtype("uint8")))
im_array = np.reshape(im_array, (image_data.height, image_data.width, 4))
im_array = im_array[:, :, :3][:, :, ::-1]

获取lidar的输出

p_cloud_size = len(lidar_data)
p_cloud = np.copy(np.frombuffer(lidar_data.raw_data, dtype=np.dtype('f4')))
p_cloud = np.reshape(p_cloud, (p_cloud_size, 4))

intensity = np.array(p_cloud[:, 3])

# Point cloud in lidar sensor space array of shape (3, p_cloud_size).
local_lidar_points = np.array(p_cloud[:, :3]).T

# Add an extra 1.0 at the end of each 3d point so it becomes of
# shape (4, p_cloud_size) and it can be multiplied by a (4, 4) matrix.
local_lidar_points = np.r_[
    local_lidar_points, [np.ones(local_lidar_points.shape[1])]]

# This (4, 4) matrix transforms the points from lidar space to world space.
lidar_2_world = lidar.get_transform().get_matrix()

Lidar的局部坐标转换为世界坐标

world_points = np.dot(lidar_2_world, local_lidar_points)

将lidar的世界坐标转换到图像坐标

world_2_camera = np.array(camera.get_transform().get_inverse_matrix())

# Transform the points from world space to camera space.
sensor_points = np.dot(world_2_camera, world_points)

- 物理引擎

physics_vehicle = vehicle.get_physics_control()

- 传感器安装

初始化车辆

bp = world.get_blueprint_library().filter('charger_2020')[0]
vehicle = world.spawn_actor(bp, random.choice(world.get_map().get_spawn_points()))
vehicle_list.append(vehicle)
vehicle.set_autopilot(True)

DisPlayManager()

self.display = pygame.display.set_mode(window_size, pygame.HWSURFACE | pygame.DOUBLEBUF)

SensorManager

传感器类

传入world，dm，传感器的类型，放置的位置，看起来仿真中所有的传感器都可以放置到同一个位置，就不存在各个坐标系转换的问题，但是观察到的内容是如何变化的呢？

SensorManager(world, display_manager, 'RGBCamera', carla.Transform(carla.Location(x=0, z=2.4), carla.Rotation(yaw=-90)), 
              vehicle, {}, display_pos=[0, 0])

Init初始化构造，事件监听，并键入到displaymanager中；

实现render函数；

def render(self):
    if self.surface is not None:
        offset = self.display_man.get_display_offset(self.display_pos)
        self.display_man.display.blit(self.surface, offset)

sufface是将渲染出来的结果处理为pygame可渲染的对象

def save_rgb_image(self, image):
    t_start = self.timer.time()

    image.convert(carla.ColorConverter.Raw)
    array = np.frombuffer(image.raw_data, dtype=np.dtype("uint8"))
    array = np.reshape(array, (image.height, image.width, 4))
    array = array[:, :, :3]
    array = array[:, :, ::-1]

    if self.display_man.render_enabled():
        self.surface = pygame.surfarray.make_surface(array.swapaxes(0, 1))

其他函数，如save image，lidar，semantilidar_image,lidar_image

- Gbuffer（当前报错-后续查看）

- 根据id获取蓝图中的实体类对象

vehicles_list = world.get_actors().filter('vehicle.*')

- 跟随actor漫游模式

spectator = world.get_spectator()

transform = vehicle.get_transform()

spectator.set_transform(carla.Transform(transform.location + carla.Location(z=50),

carla.Rotation(yaw=int(transform.rotation.yaw), roll=0, pitch=-90)))

- 绘制几何体

debugHelper =world.debug

debugHelper.draw_line(carla.Location(0,0,0),carla.Location(100,100,0))

可以draw arrow point,box等